Feng Chia University

Electrical Engineering Fundamentals I Lab

Laboratory 7

Step Response Measurement

Instructor: Prof. Shyan-Lung Lin

Student Name: 周嘉禾

Student ID: D1166506

Experiment Date:30/11/2023

- Introduction

- Display a specified step response on your oscilloscope of a specified network.

- Document the step response and accurately calculate time constants for the specified network.

- Use the step response technique to characterize an unknown capacitor or inductor which includes:

  - Selecting and analyzing the model for the device under test

  - (DUT).

  - Designing and preparing an appropriate measurement circuit.

  - Computing the capacitance or the inductance based on the time constant and analysis of the measurement circuit.

- Materials

  - Waveform Generator

  - Oscilloscope

  - Components

    - Variable Resistor RT

    - Resistors: R = 1 kΩ

Capacitor: C = 0.1µF

    - Resistors: R = 1 kΩ

Inductor: L = 4.7 mH

- Circuit diagram

▲ Figure 1. Circuit of Experiment 7.a Properties of Instruments step 1

▲ Figure 2. Circuit of Experiment 7.a Properties of Instruments step 2

▲ Figure 3. Circuit of Experiment 7.a Properties of Instruments step 3~5

▲ Figure 4. Circuit of Experiment 7.b Measurement of a Known Typical

Capacitor

▲ Figure 5. Circuit of Experiment 7.c Measurement of a Precision Inductor

- Methods

Make measurements on oscilloscope and use digital multimeter on the resistor

- Experiments data

  - Experiment 7.a Properties of Instruments

Table 1: Results of the capacitor of Oscilloscope

| Rs | Rsc | τ | Csc |
| --- | --- | --- | --- |
| 52.9020 Ω | 0.9842 MΩ | 49.0000 ns | 926.2410 pF |

  - Experiment 7.b Measurement of a Known Typical Capacitor

Table 2: Measurement of known capacitor

| RTh | τ | Ceq | C |
| --- | --- | --- | --- |
| 1052.9020 Ω | 102.6000 μs | 97.4450 nF | 96.5187 nF |

Table 3: Percentage error of measurement of known capacitor

| Measured | Theoretical | % Error |
| --- | --- | --- |
| 96.5187 nF | 100 nF | -3.48% |

  - Experiment 7.c Measurement of a Precision Inductor

Table 4: Measurement of inductor

| Req | τ | L |
| --- | --- | --- |
| 1052.902 Ω | 4.4800 μs | 4.7170 mH |

Table 5: Percentage error of measurement of inductor

| Measured | Theoretical | % Error |
| --- | --- | --- |
| 4.7170 mH | 4.7 mH | 0.36% |

- Results

▲ Figure 6. Curve of Experiment 7.a Properties of Instruments

▲ Figure 7. Curve of Experiment 7.b Measurement of a Known Typical

Capacitor

▲ Figure 8. Curve of Experiment 7.c Measurement of a Precision Inductor

- Discussion

With proper measurement, the value of capacitors and inductors can be obtained easily.

- Conclusion

On capacitors and inductors, it’s easy to observe that these two kinds of circuits can make step response and let the waves become smooth.