Feng Chia University

Electrical Engineering Fundamentals II Lab

Laboratory 3

AC RL Circuits and Phasor

Instructor: Prof. Shyan-Lung Lin

Student Name: 周嘉禾

Student ID: D1166506

Experiment Date:14/03/2023

- Introduction

- To observe the RL Circuits and Phasor under Alternative Current

- Materials

  - Waveform Generator

  - Digital Oscilloscope

  - Digital Multimeter

  - Devices

Resistors: R = 1 Ω, 10 Ω, 100 Ω, 1kΩ

Inductor: L = 1 mH

- Circuit diagram

▲ Figure 1. Circuit of Experiment 3.a Pure Inductive AC Circuit

▲ Figure 2. Circuit of Experiment 3.b RL Series Circuit

▲ Figure 3. Circuit of Experiment 3.c RL Parallel Circuit

- Methods

Using Digital Multimeter to observe current and voltage and Oscilloscope to observe the wave.

- Experiments data

  - Experiment 3.a Pure Inductive AC Circuit

Table 1: Measurement of Pure Inductive AC Circuit

| f | XL | I |  | θ |
| --- | --- | --- | --- | --- |
|  |  | Theoretical | Measurement |  |
| 100 Hz | 62.8319 Ω | 0.0796 A | 0.0090 A | 9.65° |
| 500 Hz | 314.1593 Ω | 0.0159 A | 0.0067 A | 45.98° |
| 1 kHz | 628.3185 Ω | 0.0080 A | 0.0046 A | 65.05° |
| 10 kHz | 6283.1853 Ω | 0.0008 A | 0.0018 A | 80.14° |

  - Experiment 3.b RL Series Circuit

Table 2: Measurement of RL Series Circuit

|  | VS | VR | VL | I |
| --- | --- | --- | --- | --- |
| Theoretical | 2 V | 1.98 V | 0.198 V | 1.98 mA |
| Measurement | 1.9203 V | 1.9201 V | 543 μV | 1.839 mA |

|  | XL | Z | I leads VS by |
| --- | --- | --- | --- |
| Theoretical | 99.9 Ω | 1004 Ω | lags 5.7° |
| Measurement | 0.2952 Ω | 1043.9817 Ω | lags 73.71° |

  - Experiment 3.c RL Parallel Circuit

Table 3: Measurement of RL Parallel Circuit

|  | VS | XL | Z | I leads VS by |
| --- | --- | --- | --- | --- |
| Theoretical | 2.8 V | 99.9 Ω | 99.4 Ω | lags 84° |
| Measurement | 2.24 V | 99.7284 Ω | 66.2820 Ω | lags 111.8° |

|  | I | IR | IL |
| --- | --- | --- | --- |
| Theoretical | 0.028 A | 2.8 A | 0.028 A |
| Measurement | 0.0338 A | 0.0113 A | 0.0225 A |

- Results

▲ Figure 4. Results of Experiment 3.a 100 Hz

▲ Figure 5. Results of Experiment 3.a 500 Hz

▲ Figure 6. Results of Experiment 3.a 1 kHz

▲ Figure 7. Results of Experiment 3.a 10 kHz

▲ Figure 8. Results of Experiment 3.b.2

▲ Figure 9. Results of Experiment 3.b.3

▲ Figure 10. Results of Experiment 3.c.2

▲ Figure 11. Results of Experiment 3.c.3

- Discussion

Explain why the phase lag between current and voltage for the inductor is not 90°?

Because of the impedance and resistance involved, the phase lag will not be exact 90°. But the higher the frequency is, the phase lag will be more close to 90°.

- Conclusion

From the graphs above, current will lag voltage.