Feng Chia University

Electrical Engineering Fundamentals II Lab

Laboratory 4

AC RC Circuits and Phasor

Instructor: Prof. Shyan-Lung Lin

Student Name: 周嘉禾

Student ID: D1166506

Experiment Date:21/03/2023

- Introduction

- To observe the RC Circuits and Phasor under Alternative Current

- Materials

  - Waveform Generator

  - Digital Oscilloscope

  - Digital Multimeter

  - Devices

Resistors: R = 1 Ω, 10 Ω, 100 Ω, 1kΩ

Capacitor: C = 1 μF

- Circuit diagram

▲ Figure 1. Circuit of Experiment 4.a Pure Capacitive AC Circuit

▲ Figure 2. Circuit of Experiment 4.b RC Series Circuit

▲ Figure 3. Circuit of Experiment 4.c RC Parallel Circuit

- Methods

Using Digital Multimeter to observe current and voltage and Oscilloscope to observe the wave.

- Experiments data

  - Experiment 4.a Pure Capacitive AC Circuit

Table 1: Measurement of Pure Capacitive AC Circuit

| f | XC | I |  | θ |
| --- | --- | --- | --- | --- |
|  |  | Theoretical | Measurement |  |
| 100 Hz | 1591.5 Ω | 0.889 mA | 0.923 mA | 1.62° |
| 500 Hz | 318.3 Ω | 4.443 mA | 4.425 mA | -90.05° |
| 1 kHz | 159.2 Ω | 8.886 mA | 8.324 mA | -90.24° |
| 10 kHz | 15.9 Ω | 88.858 mA | 23.062 mA | -120.79° |

  - Experiment 4.b RC Series Circuit

Table 2: Measurement of RC Series Circuit

|  | VS | VR | VC | I |
| --- | --- | --- | --- | --- |
| Theoretical | 1.414 V | 0.7072 V | 1.224 V | 7.071 mA |
| Measurement | 1.2347 V | 0.5983 V | 1.0494 V | 6.024 mA |

|  | XC | Z | I leads VS by |
| --- | --- | --- | --- |
| Theoretical | 173.1087 Ω | 199.9802 Ω | 59.9806° |
| Measurement | 174.1999 Ω | 204.963479 Ω | 57.6° |

  - Experiment 4.c RC Parallel Circuit

Table 3: Measurement of RC Parallel Circuit

|  | VS | XC | Z | I leads VS by |
| --- | --- | --- | --- | --- |
| Theoretical | 3 V | 750.7508 Ω | 600.4804 Ω | 53.1033° |
| Measurement | 2.8563 V | 731.8217 Ω | 581.0212 Ω | 111.8° |

|  | I | IR | IC |
| --- | --- | --- | --- |
| Theoretical | 4.996 mA | 3 mA | 3.996 mA |
| Measurement | 4.916 mA | 2.893 mA | 3.903 mA |

- Results

▲ Figure 4. Results of Experiment 4.a 100 Hz

▲ Figure 5. Results of Experiment 4.a 500 Hz

▲ Figure 6. Results of Experiment 4.a 1 kHz

▲ Figure 7. Results of Experiment 4.a 10 kHz

▲ Figure 8. Results of Experiment 4.b.2

▲ Figure 9. Results of Experiment 4.b.3

▲ Figure 10. Results of Experiment 4.c.2

▲ Figure 11. Results of Experiment 4.c.3

- Discussion

Explain why the phase lead between current and voltage for the capacitor is not 90°?

Because of the impedance and resistance involved, the phase lead will not be exact 90°. But the higher the frequency is, the phase lead will be more close to 90°.

- Conclusion

From the graphs above, current will lead voltage.